# Joint Control of Manufacturing and Onsite Microgrid System via Novel Neural-Network Integrated Reinforcement Learning Algorithms

Jiaojiao Yang, Zeyi Sun*, Wenqing Hu, and Louis Steimeister

The major revisions have been made in red text.

Abstract: Microgrid is a promising technology of distributed energy supply system, which consists of storage devices, generation capacities including renewable sources, and controllable loads. It has been widely investigated and applied for residential and commercial end-use customers as well as critical facilities. In this paper, we propose a joint state-based dynamic control model on microgrids and manufacturing systems where optimal controls for both sides are implemented to coordinate the energy demand and supply so that the overall production cost can be minimized considering the constraint of production target. Markov Decision Process (MDP) is used to formulate the decision-making procedure. The main computing challenge to solve the formulated MDP lies in the co-existence of both discrete and continuous parts of the high-dimensional state/action space that are intertwined with constraints. A novel reinforcement learning algorithm that leverages both Temporal Difference (TD) and Deterministic Policy Gradient (DPG) algorithms is proposed to address the computation challenge. Experiments for a manufacturing system with an onsite microgrid system with renewable sources have been implemented to justify the effectiveness of the proposed method.

Key Words: Microgrid, Manufacturing, Reinforcement Learning, Markov Decision Process, Temporal Difference Learning, Deterministic Policy Gradient

# 1. Introduction

A microgrid is a localized autonomous energy system that consists of distributed energy sources and loads [1], which can operate either separated from, or connected to, external utility power grids [2, 3]. It is considered a reliable solution to satisfy the growing demand for electric power through strengthening the resilience and mitigating the disturbances of the grid [4, 5].

Various studies on microgrids have been conducted for residential houses and critical facilities, such as medical centers, financial corporations, military bases, and jails (see details in Section 2.1). A great deal of literature focusing on the optimal control strategies of using microgrids has been reported (see details in Sections 2.2 and 2.3).

While for the manufacturing industry, the loss due to blackouts is not that direct or explicit when compared to the ones that are traditionally considered “critical”. The incurred losses such as unemployment and supply chain failure cannot be sensed and evaluated immediately after blackouts. The overall influence in term of both economic and societal aspects seems to be underestimated. Thus, the research on the application of microgrid in manufacturing has been less reported compared to residential sector.

The benefits of combining microgrids with manufacturing systems can be discussed from two aspects. On one hand, the microgrids can offer one more option of energy supply in terms of cost effectiveness. The cost of energy delivered by utility company varies depending on different time periods. The use of microgrids can provide a more flexible solution of energy supply considering the cost variation. The users can select different energy sources as well as determine energy flow to optimize their performance with respect to cost effectiveness. In addition, the use of microgrids can enhance the security of manufacturing systems against the utility failures due to various reasons (e.g., natural disaster). On the other hand, microgrids can help manufacturing practitioners further improve their performance from the perspective of environmental sustainability. The adoption of renewable sources in microgrids can significantly improve the carbon footprints of the energy supplied for production.

Therefore, this study aims to enhance the use of microgrid in manufacturing sector through building a joint control model that can simultaneously adjust the energy supply and demand from both microgrid and manufacturing sides towards cost effectiveness and environment sustainability.

Recently, with the increasing concerns on climate change and environmental protection, the benefits of integrating energy-aware strategies in manufacturing have been recognized. Many sustainability-aware and carbon-constrained operating strategies, either on individual manufacturing processes or entire manufacturing system, have been investigated and used by many practitioners (see details in Section 2.4).  Meanwhile, the use of microgrid to support manufacturing has also been launched, in particular, in the areas of benefits evaluation, system sizing, and microgrid-side control (see details in Sections 2.5 and 2.6), among which many investigations on using integrated energy systems for process industry have been reported (see details in Section 2.7).

However, the study of joint energy control for both microgrids and manufacturing systems simultaneously has not yet been fully launched. In this paper, a joint control model considering both microgrids and manufacturing systems is established using Markov Decision Process (MDP). A novel reinforcement learning algorithm is proposed for solving the MDP. The algorithm uses both Temporal Difference (TD) method to search for the discrete optimal control actions and Deterministic Policy Gradient (DPG) algorithms to search for the continuous optimal control actions, together with function approximations of the action-value function via a neural network. Experiments based on a manufacturing system with an onsite microgrid with renewable sources are implemented under real parameters to identify optimal control actions for both manufacturing system and microgrid towards cost optimality. It is empirically validated that the optimal policies found by our reinforcement learning algorithm are more efficient in production and incur less cost when compared to randomly sampled policies and a routine operation policy. Last but not least, due to its exploration-exploitation nature, the proposed reinforcement learning method is also effective in the case when the MDP parameters are not known but have to be learned during the algorithm dynamics. The proposed MDP model as well as the reinforcement learning algorithms can well address the major challenges with respect to modeling and problem solving as follows.

On one hand, the combined system including both manufacturing and microgrid has a complex interaction when controls are implemented from both sides. For example, the controls on manufacturing systems will influence the energy demand that needs to be met through controlling the operations of microgrids as well as the external utility connections to achieve an energy flow balance. The manufacturing system itself is a complex system where the interrelationships among different machines in the system need to be quantified. The manufacturing throughput should also be cared when energy control for the manufacturing system is implemented. All these factors need to be carefully considered.

On the other hand, it can be expected that the space of the states and the actions in the MDP model could be extremely large, which makes most existing strategies for solving MDP such as reinforcement learning algorithms less effective. An initial study by authors has shown that a traditional algorithm, e.g., vanilla Q-learning, integrated with a neural network can only work for a small sized model, while cannot sufficiently address the model with a large space size [6].

In summary, the major contributions of the proposed joint control model are summarized as follows:

- In most relevant literature, the control is only focused on the microgrid side, while neglecting the demand side (i.e., manufacturing system). In some research where manufacturing side is integrated into the control scheme, the manufacturing system is usually simplified and thus the complex internal dynamics cannot be fully represented (see details in Section 2.6). In this paper, the dynamic joint controls of manufacturing systems and microgrids, e.g., the complex dynamics in the demand side when energy control is implemented on manufacturing systems, are modeled.

- In existing literature, the renewable sources are usually modeled as the system state in MDP, that means, the renewable energy is dependent on the variations of wind and solar sources. It ignores the opportunities of controlling the energy supply from the renewable sources. In this paper, the availability of renewable sources is modeled as the state, while the ON/OFF control of renewable generation capacity is modeled as the control actions.

- There exist a few published works that use reinforcement learning algorithms such as deep Q-learning (DQN) to solve dynamic decision-making problems with respect to microgrid operations [7, 8]. However, Q-learning algorithms is typically constrained by problem size, and thus cannot work very well if the problem size is too large. Our novel reinforcement learning algorithm integrates DPG with TD to treat the co-existence of discrete and continuous states and actions. We also address the constraints via proximal projection operators and policy gradient updates.

The remaining part of the paper is organized as follows. A brief literature review in several relevant areas is given in Section 2. Section 3 introduces the formulation of the dynamic decision-making model using MDP in detail. Section 4 proposes our reinforcement learning algorithm. Section 5 implements numerical case studies through extensive experiments and sensitivity analysis to validate the effectiveness of the proposed method. Section 6 concludes the paper and discusses future works. Supplementary materials in terms of mathematic derivation are summarized in Section 7.

# Literature Review

2.1 Microgrids for Residential Properties, Community Services, and Critical Facilities

The research of microgrid applications in residential properties, community services, and critical facilities has been widely reported in the literature. For example, Faruque discussed an economically profitable way to deploy a residential microgrid incorporating a new market entity that can act as both a consumer and a power supplier [9]. Kriett and Salani proposed a generic mixed integer linear programming model to minimize the operating cost of a residential microgrid [10]. Roggia et al designed a sustainable residential microgrid system including PHEV and energy storage devices [11]. Ahourai and Faruque analyzed the impact of a residential microgrid under various electric vehicle penetration levels [12]. Igualada et al. proposed an optimization model to manage a residential microgrid including a charging spot with a vehicle-to-grid system and renewable energy sources [13]. Hawkes and Leach investigated the cost-effective operating strategy for residential micro-combined heat and power (CHP) systems [14]. Tasdighi et al. investigated a micro-CHP based residential microgrid scheduling problem using smart meter data and temperature dependent thermal load modeling [15]. Kakigano et al. applied a direct-current microgrid to residential houses with a cogeneration system, such as a gas engine or fuel cell [16].

2.2 Microgrid Control Strategies Using Traditional Methodologies

Considering the complex dynamics, in particular, the uncertainties of the renewable sources and the load, involved in microgrid operation, Model Predictive Control (MPC)-based approaches have been widely used to estimate the uncertainty and offer an optimizer to solve the best schedules of microgrid operations [17]. For example, an online optimal energy management model for energy storage system in a microgrid was developed using a mixed integer linear programming (MILP) over a rolling horizon period [18]. A similar method considering time-varying constraints of a microgrid was proposed [19]. MPC was applied to the operation of a hydrogen-based hybrid energy storage system in a microgrid [20]. A robust optimization model was proposed for microgrid operation using ensemble weather forecasts [21].

Another major research approach to address the challenges of uncertainties is stochastic optimization. For example, a stochastic energy scheduling model in microgrids with intermittent renewable energy resources was proposed [22]. A risk-averse stochastic programming method was proposed, which considered not only the expectation but the variation of the total cost [23]. A two-stage stochastic programming approach with MPC for microgrid energy management was proposed considering the uncertainty of load demand, renewable energy generation, and electricity prices [24]. From the perspective of control objectives, the existing literature focuses on optimizing cost [25, 26], power system stability [27, 28] such as voltage frequency control [29, 30], and environmental objectives such as reducing carbon dioxide emissions [31, 32].

2.3 Microgrid Control Strategies Using Machine Learning Methodologies

Recently, some researchers have started to leverage the methodologies such as deep learning and reinforcement learning to address the microgrid control problem, i.e., reinforcement learning framework is used to solve the sequential decision-making problem typically formulated by MDP, and deep learning is used to approximate the values of some critical components in MDP such as Q-value and the state value. For example, Zeng et al. presented a novel dynamic energy management tool that incorporates efficient management of energy storage system into microgrid real-time dispatch while considering power flow constraints and uncertainties in load, renewable generation, and real-time electricity price [7]. Ji et al. employed a deep reinforcement learning approach to identify the optimal control strategy of microgrids that can minimize the daily operating cost. In their study, a deep feed-forward neural network is designed to approximate the optimal action-value function, and the deep Q-network algorithm is used to train the neural network [8].

2.4 Sustainability-Aware and Carbon-Constrained Manufacturing Control Strategy

Manufacturing activities dominate energy consumption and green-house-gas (GHG) emissions in the industrial sector [33] that accounts for one third of the total energy consumption in the U.S. [34]. Thus, many studies focusing on building energy consumption framework and exploring emissions for various industries have been reported. For example, a big data driven analytical framework was proposed to reduce the energy consumption and emission for energy-intensive industries [35, 36]. Therblig power model were developed for calculating the energy supply of computer numerical control (CNC) machine tools using machining process parameters [37]. Yun et al. proposed a CPS-enabled and knowledge-aided demand response strategy for sustainable manufacturing [38]. Later, the team investigated a demand response strategyfor manufacturing systems considering the implications of fast-charging battery powered material handling equipment [39].

In addition, many sustainability-aware and carbon-constrained operating and control strategies have been investigated for manufacturing. Optimal energy control towards a smart and sustainable manufacturing paradigm has been widely reported. For example, work-in-process parts in manufacturing systems were utilized for optimal production control to reduce energy cost without sacrificing production throughput [40]. A joint energy control and maintenance scheduling model was proposed to minimize overall operational costs considering time-dependent energy cost as well as equipment degradation [41].

  - Microgrid System Sizing for Manufacturing

The benefits of using microgrids in manufacturing has been gradually recognized by both academic colleagues and industrial practitioners. It is hardly possible to maintain manufacturing operations without electricity supply nowadays, even a very short power outage can lead to detrimental impacts on manufacturing companies [42, 43]. Many studies focusing on the optimal design and component sizing of the microgrid for manufacturing plant has been reported [44-46]. For example, a Mixed Integer Non-Linear Programming optimization model was proposed for sizing the capacity of onsite generation system with renewable sources and battery energy storage system for the manufacturers considering the energy loads from both the manufacturing system and HVAC (Heating, Ventilation, and Air Conditioning) system in a typical manufacturing plant [44]. A time series model was proposed to describe and predict the variation of the energy load of manufacturing systems and the irradiation of solar energy such that a case study examining the cost for building and running an onsite microgrid system considering different microgrids capacities was implemented [46]. Santana-Viera et al. estimated the on-site wind and solar power capacity that could maximize the cost saving of a manufacturing facility in the interruptible load demand response program [47].

  - Microgrid Control for Manufacturing

Some studies on using microgrid for manufacturing have been recently reported. For example, Harper et al. proposed a demand-side management method for manufacturing that uses a microgrid to reduce energy consumption and improve system reliability [48, 49]. They employed a simple rule-based control strategy to adjust the operation modes of industrial pumps based on the empirical parameters of supply capability and electricity demand. Golari et al. presented a multi-period, production-inventory planning model in a multi-plant manufacturing system powered with onsite and grid renewable energy to determine the production quantity, the stock level, and the renewable energy supply in each period such that the aggregated production cost (including energy) is minimized [50]. It is modeled as a static optimization problem, rather than a dynamic state-based decision making.

The common limitations of the existing studies on microgrid control for manufacturing can be summarized as follows. 1) Manufacturing system is usually simplified and modeled as a single isolated machine, which ignores the complex dynamics and interactions of various equipment within the manufacturing system. 2) The manufacturing side is typically considered a fixed load, while neglecting its control flexibility. 3) The control strategies used for the manufacturing equipment are usually simple heuristic and/or empirical rule based utilizing the empirical parameters of supply capability and electricity demand.

2.7 Integrated Energy Systems in Process Industry

Process industries, e.g., food, pulp and paper, basic chemicals, refining, and iron/steel, are typically considered to be energy intensive. For example, the electricity demand of pulp & paper industry in 2020 was estimated to be 2.08EJ [51]. Many studies in terms of improving energy efficiency and reducing greenhouse gas emission have been reported. Pandey and Prakash systematically analyzed Energy Conservation Opportunities in Pulp & Paper Industry [52]. Ashok proposed a peak load management model for steel plant [53]. Later, he and his colleague generalized the previous reseach to an optimization mode for industrial load management [54]. Ma et al. presented an architecture of energy-cyber-physical system enabled management for energy-intensive process industries to enhance the implementation of integrated energy system towards a cleaner production strategy [55]. Later, this team proposed a big data-driven analytical framework for energy-intensive process industries towards sustainability [56, 57]. Zhang et al. explored the physical and chemical characteristics of fugitive emission sources in the iron and steel production process [58]. Sun et al. systematically analyzed the integrated optimization of material and energy flows in the iron and steel industry [59]. Zhang et al. proposed a carbon flow tracing and carbon accounting method for exploring CO2 emissions of the iron and steel industry [60].

# 3. Markov Decision Process (MDP) for Joint Control of Manufacturing and Onsite Microgrid Systems

## 3.1 Formulate Joint Energy Control Problem Using Markov Decision Process (MDP)

3.1.1 Modeling Microgrids and Manufacturing System

A Markov Decision Process (MDP) model is proposed to model the decision-making of the joint control of both on-site microgrid generation system and manufacturing system. MDP has been widely used in modeling complex and evolving state-based decision-making problems [61, 62]. In this paper, the microgrid system used is a typical setup consisting of a gas turbine generator, a battery bank as well as solar PV modules and wind turbines as shown in Figure 1. Note that, traditionally, microgrid means a distributed energy generation system and renewable sources are not the default components. While, in recent years, with the increasing penetration and popularity of renewable sources in energy systems, the integration of wind and solar energy in microgrid has become the main stream in literature when studying microgrids, especially, considering the benefits with respect to environmental sustainability. Thus, renewable components are considered the default components in the microgrids modeled in this model.

The manufacturing system modeled is a typical serial production line with N machines and N-1 buffers as shown in Figure 2 (where N=5, and work-in-progress parts are stored in buffers). Let i=1,...,N be the indexes of the machines and i=1,...,N-1 be the indexes of the buffers.

Figure 1. A microgrid with various components

Figure 2. A typical manufacturing system with N machines and N-1 buffers (here N=5)

Note that the type of the manufacturing system modeled in the paper is a flow shop setting. Many industries use such a layout for their production system, e.g., auto assembly system, aeroplane assembly system, etc. In practice, the system may be more complex, say, with some parallel configurations, while, a serial line is considered a fundamental layout from which many complex layouts can be studied through appropriate extensions.

The time horizon is discretized and divided into a set of discrete intervals, with the actual time duration for each interval to be Δt. The time variable t denotes the indexes of the decision epochs of such discrete intervals at which the control actions identified based on the optimal policy and the given states can be implemented. The state, policy, state transition, objective function, and constraints of the proposed MDP are introduced as follows.

3.1.2 System State

Let the system states form a state space S. The system state at decision epoch t is denoted by St. It includes the states of manufacturing system (Stmfg), microgrid system (Stmic), and exogenous environmental features (Stenv), which can be formulated by St=Stmfg,Stmic,Stenv. Stmfg can be denoted by Stmfg=S1tM,...,SNtM,S1tB,...,SN-1tB, where SitMi=1,...,N denotes the state of machine i in the manufacturing system at decision epoch t; SitBi=1,...,N-1 denotes the state of the buffer i in the manufacturing system at decision epoch t. Machine states include operational, blockage, starvation, off, and breakdown. Blockage means that the machine itself is not failed while the completed part cannot be delivered to the downstream buffer due to the breakdown of specific downstream machines. Starvation means that the machine itself is not failed while there is no incoming part from the upstream buffer due to the breakdown of specific upstream machines. The set of machine states is thus {Opr,Blo,Sta,Off,Brk}, where Opr,Blo,Sta,Off and Brk denote the operational state, blockage state, starvation state, off state, and breakdown state, respectively. At each state, there is a corresponding power consumption state, which can be illustrated by Figure 3.

Figure 3. Operation and energy states.

Stmic can be denoted by Stmic=gts,gtw,gtg,SOCt , where gts,gtw, and gtg denote the working status of solar PV, wind turbine, and generator, respectively, of the onsite microgrid generation system at decision epoch t (working=1, not working=0). A non-negative real number SOCt, denotes the state of charge of the battery system at decision epoch t.

The state describing exogenous environmental features, Stenv, can be denoted by Stenv=It,vt , where It denotes the solar irradiance at decision epoch t, and vt denotes the wind speed at decision epoch t. The exogenous feature has an impact on the system dynamics and the cost function, but cannot be influenced by the control actions. The feature is time and weather dependent. The model formulation considers the availability of a deterministic forecast of the exogenous state information. The states It,vt are taken from one year’s data (assumed to be 360 days and 24 hours/day, so in total 8640 hours).

3.1.3 Control Actions and Policy

All admissible actions constitute an action space A. Let π be the policy that maps from different states S to the actions A. The control actions adopted at decision epoch t can be denoted by At. It includes the control actions for the manufacturing system (Atmfg) and the microgrid system (Atmic), which can be denoted by At=Atmfg,Atmic. Atmfg can be denoted by Atmfg=at1,...,atN, where ati i=1,...,N is the control action for machine i at the decision epoch t. The actions include K-action, W-action, and H-action. K-action intends to keep original machine states, which can be applied to the machines in Opr,Blo,Sta,Off, and Brk states (note that machine repair is not considered a control action in this paper and repair is assumed to be a supposed-to-be reaction, so K-action used for breakdown machine can imply that repair will be implemented). H-action intends to turn off the machine, which can only be applied to the machine in Opr,Blo, and Sta states. W-action intends to turn on the machine that was previously turned off, which can only be applied to the machine in Off states. Note that for model simplicity, we assume that the energy consumption and time required for the transitions between different machine states can be ignored. The time and energy required for state transition depend on machine characteristics. For example, many CNC (Computer Numerical Control) machines have a relatively short switch time as well as lower switch energy consumption when machine state is adjusted.

Atmic specifies the actions with respect to the adjustment of the working status of the components of the microgrid as well as the corresponding energy flow and allocation in the joint system, which can be denoted by Atmic=ats,atw,atg,stm,stb,stsb,wtm,wtb,wtsb,gtm,gtb,gtsb,ptm,ptb,btm. Here ats,atw, and atg are the actions of adjusting the working status (i.e., 1 is connected, 0 is not connected to the load) of the solar, wind, and generators in microgrids; stm,stb, and stsb denote the solar energy used for supporting manufacturing, charging battery, and sold back to grids, respectively. Similarly, the notations w and g with corresponding superscripts denote the allocation of the energy generated by wind turbine and generator, respectively. ptm and ptb denote the use of the energy purchased from the grid, i.e., for supporting manufacturing, and charging battery, respectively. Note that the energy purchased from the grid is not considered for sold back due to utility policies. Finally, btm denotes the energy discharged by the battery for supporting manufacturing, and is given by a binary variable δtbm, so that btm=b⋅δtbm⋅Δt for some discharging rate b>0. Note that the energy discharged by the battery is not considered for sold back.

3.1.4. State Transition

Let the function P: S×A×S →[0,1] be the transition probability function, so that PS',A,S≡PrS'|S,A is the probability of transition to state S' given that the previous state was S and action A was taken. It is assumed that the state transition happens at the beginning of each interval when the decision is made.

For the manufacturing system, the buffer state at decision epoch t+1 can be obtained by (1) based on the states and the control actions adopted at decision epoch t of upstream and downstream machines:

| Sit+1B=SitB+ISitM,ait-ISi+1tM,ai+1t,0≤SitB≤Ni, | (1) |
| --- | --- |

Here Ni is the capacity of buffer i, and ISitM,ait is an indicator function that is defined by (2):

| ISitM,ait= 1 ,when SitM=Opr and ait=K 0 , when SitM≠Opr or ait=H | (2) |
| --- | --- |

Referring to the literature focusing on the statistical methods for machine reliability [63], we assume Li, which is the random lifetime of machine i, follows Weibull distribution with specific shape parameter and scale parameter. The probability that machine i goes into breakdown or non-breakdown state at the next decision epoch t+1, given it is not in breakdown state at the current decision epoch t can be described by (3) and (4) respectively:

| PrSit+1M=Brk¦SitM≠Brk,SitM≠Off=PrLi<t+Δt | (3) |
| --- | --- |
| PrSit+1M≠Brk¦SitM≠Brk,SitM≠Off=PrLi≥t+Δt | (4) |

Whether the machine is at Off state or not at next decision epoch can be determined in a deterministic way by (5):

| Sit+1M=Off if SitM=Off and ait=K or SitM≠Off and ait=H | (5) |
| --- | --- |

In addition, we also assume Di, which is the random repair time of machine i, follows Exponential distribution [64]. The probability that machine i completes or does not complete the repair at the next decision epoch t+1, given it is in repair at the current decision epoch t can be described by (6) and (7) respectively.

| PrSit+1M≠Brk¦SitM=Brk=PrDi<t+Δt | (6) |
| --- | --- |
| PrSit+1M=Brk¦SitM=Brk=PrDi≥t+Δt | (7) |

From (3)-(7) we can decide whether the machine state Sit+1M is Brk or Off or any one of the states Sta, Blo, Opr. To further determine the exact state if we fall into one of the states Sta, Blo, Opr, we use the following rules. For 2≤i≤N:

| Sit+1M=Sta if Sit+1M≠Brk/Off and Si-1t+1B=0 and Si-1t+1M=Brk/Sta/Off | (8) |
| --- | --- |

and for 1≤i≤N-1:

| Sit+1M=Blo if Sit+1M≠Brk/Off and Sit+1B=Ni and Si+1t+1M=Brk/Blo/Off | (9) |
| --- | --- |

and for all 1≤i≤N:

| Sit+1M=Opr if Sit+1M≠Brk/Sta/Blo | (10) |
| --- | --- |

Notice that (8) and (9) can be used to find all the Sta and Blo state machines by forward/backward scanning of all machines starting from the first/last machine, due to the trivial fact that S1t+1M≠Sta and SNt+1M≠Blo. With these, (10) further helps to determine Opr state for machines.

Therefore, the system operation state transition between the current decision epoch and the next decision epoch can be calculated by using (1)-(10) when Atmfg is adopted based on a given Stmfg.

For the microgrid system, the state transition of solar PV is determined by the action adopted. While, the state transition of wind turbine is determined by the action adopted and the variation of the wind speed. They can be formulated by (11) and (12), respectively:

| gt+1s=1, if at+1s=1 0, if at+1s=0 | (11) |
| --- | --- |
| gt+1w= 1, if at+1w=1 and vci≤vt+1≤vco 0, if at+1w=0 or vt+1>vco or vt+1<vci | (12) |

where vci and vco are the cut-in and cut-off wind speeds (m/s), respectively,

The state transition of generator is determined by the control actions adopted, which can be formulated by (13):

| gt+1g=1, if at+1g=1 0, if at+1g=0 | (13) |
| --- | --- |

The state transition of battery (i.e., SOC) is determined by the charging and discharging occurring between t and t+1 as well as the original SOC, which can be formulated by (14).

| SOCt+1=SOCt+stb+wtb+gtb+ptbη-btm/η | (14) |
| --- | --- |

where η is charging/discharging efficiency.

3.1.5. Objective Function

The objective is to identify an optimal policy based on the given state that can minimize the incurred cost from time t to the end of decision horizon. At time t, the cost at state St when action At is taken, can be defined by ESt,At. The total incurred cost from time 0 to the end of planning horizon, starting from state S and under policy π: S → A, is given by

| Cπ=E[t=0∞γtESt,πSt¦S0=S] | (15) |
| --- | --- |

Here γ∈[0,1) is the discount factor. The objective is to identify an optimal policy π*=argminπCπ over all policies satisfying the constraints, that can guide the decision maker to find appropriate actions based on the given system state to minimize the total incurred cost Cπ in (15).

For our model, the cost when action A is taken at state S is given by ES,A, which is equal to energy consumption cost plus the microgrid operational cost, minus production throughput reward and the sold back reward. So, it can be calculated by

| ES,A=TFS,A+MCS,A-TPS,A-SBS,A | (16) |
| --- | --- |

where TFS,A is the cost for the energy purchased from the grid, MCS,A is the operational cost for the onsite generation system, TPS,A is the reward of production throughput of the manufacturing system, and SBS,A is the sold back benefit.

Note that in this paper, since the throughput modeling and quantification for a typical manufacturing system is still a major research challenge in the field of production system engineering when considering non 100% reliable machines and finite capacity buffer [65, 66], the concern of production throughput maintaining is represented as a monetary reward and integrated into the objective function. This strategy circumvents the challenges of modeling throughput as a major constraint in MDP.

In (16), TFS,A can be calculated by

| TFS,A=pt⋅rtc=ptm+ptb⋅rtc | (17) |
| --- | --- |

where pt is the energy consumption purchased from the grid at decision epoch t, and rtc is the rate of energy consumption charge.  pt can be calculated by

| pt=Etmfg-stm+wtm+gtm+btm | (18) |
| --- | --- |

where Etmfg is the total energy consumed by the manufacturing system at decision epoch t which can be determined by:

| Etmfg=i=1NPCit⋅Δt | (19) |
| --- | --- |

where PCit is the amount of power drawn by the machine i from t to t+1. PCit can be calculated by

| PCit=0 , if SitM=Brk or SitM=Off PCiOpr, if SitM=Opr PCiIdl, if SitM=Sta or SitM=Blo | (20) |
| --- | --- |

where PCiOpr and PCiIdl are the power level of machine i at the states of Opr and Sta/Blo, respectively.

MCS,A can be calculated by

| MCS,A= ets⋅romcs+etw⋅romcw+etg⋅romcg +btm+stb+wtb+gtb⋅Δt2eSOCmax-SOCmin⋅romcb , | (21) |
| --- | --- |

where ets, etw , and etg are the energy generated at decision epoch t from the onsite solar PV, wind turbine, and generator, respectively, which are calculated from the states and actions, that will be specified below. romcs, romcw, and romcg are the unit operational and maintenance cost for generating electricity from solar PV, wind turbine, and generator respectively. romcb is the operational and maintenance cost for battery storage system per unit charging/discharging cycle. e is the capacity of battery storage system. SOCmax and SOCmin are maximum and minimum allowed state of charge of battery storage system, respectively. Note that the fraction part of the 4th term on the right-hand side of (21) represents for the fraction of the charging/discharging cycle of the battery in one decision period [45]. Specifically, the numerator calculates the total energy flow (either charging or discharging) through the battery system in one decision period, while the denominator calculates the total energy flow of one charging/discharging cycle.

ets can be calculated according to

| ets= 0 , if gts=0 It⋅a⋅δ⋅Δt/1000, if gts=1 | (22) |
| --- | --- |

where It is the solar irradiance of a certain location (W/m2) at decision epoch t, a is the area of the solar PV system, and δ is the efficiency of the system.

etw can be calculated according to [45].

| etw= 0, if gtw=0 or vt<vci or vt>vco Nw⋅RPw⋅Δt, if gtw=1 and vr≤vt<vcoNw⋅RPw⋅vt-vcivr-vci⋅Δt, if gtw=1 and vci≤vt<vr | (23) |
| --- | --- |

where vt is the wind speed (m/s) at decision epoch t. vr is the rated wind speeds (m/s). Nw is the number of wind turbine equipped within the onsite generation system and RPw is the rated power of the wind turbine (kW). RPw is determined by

| RPw=12⋅ρ⋅π⋅r2⋅vavg3⋅θ⋅ηt⋅ηg/1000 , | (24) |
| --- | --- |

where ρ is the density of air. vavg is average wind speed. θ is the power coefficient. r is the radius of the wind turbine blade. ηt is its gearbox transmission efficiency. ηg is electrical generator efficiency.

etg can be calculated by

| etg=0, if gtg=0ng⋅Gp⋅Δt, if gtg=1 | (25) |
| --- | --- |

where ng is the number of generators and Gp is the rated output power of the generator (kW).

TPS,A can be determined by

| TPS,A=prt⋅rp | (26) |
| --- | --- |

where prt is the production count at decision epoch t and rp is the unit reward for each unit of production. prt can be calculated by

| prt=1, if SNtM=Opr and atN=K 0, if SNtM≠Opr or atN=H | (27) |
| --- | --- |

Note that, in such a serial production system, the system throughput is counted by the production count of the last machine in the line since one finished product is only counted when the process of the last machine is completed. Thus, the subscript in (27) is N.

Similarly, the sold back reward SBS,A can be calculated by

| SBS,A=st⋅rsb , | (28) |
| --- | --- |

where st=stsb+wtsb+gtsb is the sold back energy to the grid at decision epoch t and rsb is the unit reward from sold back energy.

## 3.2 Parameterization of the Action Space and Constraints

Our model contains the following constraints for the action space A, that are described below.

Since we set the battery state of charge level needs to be maintained within a given range, which can be formulated by

| SOCmin≤SOCt+stb+wtb+gtb+ptbη-btmη≤SOCmax . | (29) |
| --- | --- |

Notice that (29) indicates that the actions to the microgrid on battery charging/discharging are controlled by the current SOC state.

Actions that can be applied to machines are restricted by the current machine states: K-action can be applied to the machine at Opr,Blo,Sta,Off and Brk states; H-action can only be applied to the machine at Opr,Blo and Sta states; W-action can only be applied to the machine at Off states, i.e.,

| ati=K , if SitM=Opr,Blo,Sta,Off,Brk H , if SitM=Opr,Blo,Sta W , if SitM=Off | (30) |
| --- | --- |

The energy flow balance for the energy generated by solar PV, wind turbine, and generator can be formulated by (31)-(33), respectively.

| stm+stb+stsb=ets , | (31) |
| --- | --- |
| wtm+wtb+wtsb=etw | (32) |
| gtm+gtb+gtsb=etg | (33) |

Notice that according to (22), (23) and (25), the energies ets,etw,etg depend on the working states of the microgrid gts,gtw,gtg, the solar irradiance It and the wind speed vt. The constraints (31)-(33) are restricting the actions applied to microgrid based on the current microgrid state and the environmental features.

The battery cannot be charged and discharged simultaneously. The charge/discharge constraint is represented as follows:

| stb+wtb+gtb+ptb⋅btm=0 . | (34) |
| --- | --- |

Notice that since btm=δtbm⋅b⋅Δt, we only seek for binary choices of δtbm=0/1 when stb=wtb=gtb=ptb=0.

The energy sold back to the grid and the energy purchased from the grid cannot happen simultaneously, which can be represented by

| stsb+wtsb+gtsbptm+ptb=0 . | (35) |
| --- | --- |

If the constraint (35) is satisfied at stsb=wtsb=gtsb=0, so that we allow ptm+ptb≠0, then due to supply-demand balance principle, the energy purchased from the grid should be equal to the energy consumed from the grid, so we have

| ptm+ptb=pt1{pt>0} , | (36) |
| --- | --- |

where pt is given by (18) .

To simplify the model, we further assume that the energy purchased from the grid can be used either only for supporting manufacturing or charging battery, but not simultaneously, i.e.,

| ptm⋅ptb=0 . | (37) |
| --- | --- |

Due to (35) and (37), if stsb=wtsb=gtsb=0, we can introduce a binary variable δtpb=0/1 (0 means purchased energy is not used for battery charging, 1 means purchased energy is used for battery charging) so that ptm=1-δtpbpt1{pt>0} and ptb=δtpbpt1{pt>0}; if else, i.e., any of stsb,wtsb or gtsb is not equal to zero, we have ptm=ptb=0.

To facilitate the design of policy-gradient related algorithms for training, we will further parameterize the control actions stm,stb,stsb,wtm,wtb,wtsb,gtm,gtb,gtsb by introducing proportionality parameters.

| θ=λsm,λsb,λwm,λwb,λgm,λgb | (38) |
| --- | --- |

such that the proportionality relation holds as

| stm=ets⋅λsm ,stb=ets⋅λsb ,stsb=ets⋅1-λsm-λsbwtm=etw⋅λwm ,wtb=etw⋅λwb ,wtsb=etw⋅1-λwm-λwbgtm=etg⋅λgm ,gtb=etg⋅λgb ,gtsb=etg⋅1-λgm-λgb | (39) |
| --- | --- |

These representations further simplify the constraints (31)-(33) into the following constraints

| λsm≥0,λsb≥0,0≤λsm+λsb≤1 | (40) |
| --- | --- |
| λwm≥0,λwb≥0,0≤λwm+λwb≤1 | (41) |
| λgm≥0,λgb≥0,0≤λgm+λgb≤1 | (42) |

To deal with the constraints (34), (35), and (37), we further introduce the binary (0/1) variables δtb=1{stb+wtb+gtb>0} , δtsb=1{stsb+wtsb+gtsb>0}, and δtp=1{ptm+ptb>0}. Then constraints (34), (35), and (37) become a discrete constraint

| δtb,δtp,δtpb,δtsb,δtbm∈ {1,1,1,0,0,1,1,0,0,0,1,0,0,1,0,0,1,1,0,0, 0,1,0,0,1,0,0,0,1,1} . | (43) |
| --- | --- |

Notice that (43) summarizes all discrete constraints for the control parameters on the microgrid. The remaining continuous constraints for the microgrid are only (29) and (40)-(42).

Based on (43), we can further write the constraints (29) and (40)-(42) into different constraints on θ (the parameter in (38)) and SOC. These constraints are formulated in (S1)-(S6) in Supplementary Material 7.3. All effective constraints for the admissible actions in this problem are (30), (43) and (S1)-(S6).

# 4. Novel Neural-Network Integrated Reinforcement Learning Algorithms for the MDP model

## 4.1 Abstract Formulation of the MDP Model

A state St∈S in the space of the model consists of two parts St=Std,Stc: the discrete part

| Std=S1tM,...,SNtM,S1tB,...,SN-1tB,gts,gtw,gtg,It,vt | (44) |
| --- | --- |

which consists of the machine, buffer and microgrid states, as well as the coarse-grained solar irradiance and the wind speed It,vt. Here to reduce complexity, an approximate coarse-grained scheme will be applied to each pair of the values It,vt, so that they will be approximated by integers closest to them on a grid with 20×20 states, thus taken values among 20×20 different states; the continuous part

| Stc=SOCt | (45) |
| --- | --- |

which consists of the SOC state.

An action At in the action space A of the model also consists of three parts At=Atd,Atc,Atr: the discrete part

| Atd=at1,...,atN,ats,atw,atg,δtb,δtp,δtpb,δtsb,δtbm | (46) |
| --- | --- |

which consists of the actions on each of the machines and the connecting/disconnecting action of the solar PV, the wind turbine, and the generator, as well as the indicator variables in (43); the continuous part

| Atc=stm,stb,stsb,wtm,wtb,wtsb,gtm,gtb,gtsb | (47) |
| --- | --- |

which consists of the solar/wind/generator energy used for supporting manufacturing, charging battery, and sold back to the grid. The continuous action Atc depends on St and the proportionality parameters in θ introduced in (38), i.e., Atc=Acθt,St; the remainder part Atr=ptm,ptb,btm, which consists of the use of the energy purchased from the grid for supporting manufacturing and charging battery, and the energy discharged by the battery for supporting manufacturing. These can be calculated directly from δtpb and δtbm as well as (36) and (18), which then can be calculated from Atc.

At a specific state St∈S, the actions that can be taken are restricted by this particular state via the restriction

| Atd∈DdStd | (48) |
| --- | --- |

and

| θ∈DcStd,Stc,Atd | (49) |
| --- | --- |

where Dd is the set of admissible discrete actions Atd that can be taken at the current state, and Dc is the set of admissible parameters θ for continuous actions Ac that can be taken at the current state. According to the discussions in Section 3.2, we see that Dd is given by (30) and (43), and depends only on the discrete part of the current state (actually, only on the current states of the machines) and Dc is given by one of (S1)-(S6) in Supplementary Material 7.3, that depends on both the continuous and the discrete parts of the current state, as well as the discrete actions.

## 4.2 A Brief Overview of Reinforcement Learning: Temporal-Difference and Policy Gradient methods

For an infinite horizon, discounted Markov Decision Process (MDP) with state space S and action space A, Reinforcement Learning algorithms [67] design an “agent" interacting with an environment over a number of time steps. At each time step t when the agent is at a state St, it selects an action At using the policy πSt∈ A, and then moves to the next state St+1 according to the transition probability PSt+1,At,St=PrSt+1|St,At. During this step the agent incurs a cost ESt,At. The goal of the agent is to find a policy π* that minimizes the expected total discounted cost

| π*=argminπCπ | (50) |
| --- | --- |

where Cπ≡Et=0∞γtESt,At|S0=S, and γ∈(0,1] is the discount factor. To solve (50), we introduce the action-value function Qπ(S,A), which is the expected total discounted cost under policy π starting from state S and taking initial action A. For the optimal policy π* in (50) we have Q*S,A≡Qπ*S,A=minπQπS,A. The key idea in action-value methods is to identify π* through a sequence of approximators QS,A built via an iterative scheme that approaches Q*S,A. In practice we use a function approximator, such as a neural network QS,A;ω (sometimes referred as the “critic”, see [67-70]) to replace the actual action-value function QπS,A, where ω is the parameter of the approximating function. Given current ωt, we construct the temporal difference

| δt=ESt,At+γQSt+1,At+1;ωt-QSt,At;ωt | (51) |
| --- | --- |

where St+1 is the next state of MDP from state St under action At and At+1 is selected as the optimal policy (or near-optimal policy), such as ε-greedy policy [67] at state St+1 when the approximator is given by the parameter ωt. This class of action-value methods are called (on-policy) Temporal Differences (TD or SARSA, see [67]). Actually, there are two kinds of reinforcement learning methods: on-policy and off-policy. On-policy methods attempt to evaluate or improve the policy that is used to make decisions, whereas off-policy methods evaluate or improve a policy different from that used to generate the data (see [67, Section 5.4, p. 82]). In our temporal-difference method, we are using the policy obtained from data in the current iteration step to generate new data and we repeat this iteration. So, our TD method is on-policy. Once the temporal differences (51) are constructed, we can update the critic parameter ωt according to a standard gradient descent

| ωt+1=ωt-ηωδt∇ωQSt,At;ωt , | (52) |
| --- | --- |

where ηω>0 is the learning rate. Another way to solve (50) works in the scenario when the policy can be parameterized by a policy parameter θ, so that π=πθ, Cπ=Cπθ=Cθ. In that case, we can simply use a gradient descent to find the optimal policy parameter θ* such that π*=πθ*:

| θt+1=θt-ηθ∇θCθ | (53) |
| --- | --- |

where ηθ>0 is the learning rate. The gradient ∇θCθ can be estimated by the well-celebrated policy gradient theorem when the policy πθ is random (see [67]). However, if we are restricted only to deterministic policies, so that we assume that πθS=Aθ is a deterministic policy, and thus Cθ=CAθ, then the Deterministic Policy Gradient (DPG, see [75/68]) can also be calculated as

| ∇θCθ = ES∼ρπ¦π=Aθ∇θAθ∇AQπS,A¦π=Aθ,A=Aθ | (54) |
| --- | --- |

where ρπ|π=Aθ=ES0t=1∞γtPrSt=S|S0,π is the discounted state distribution when π=Aθ.

## 4.3 Neural-Network Integrated Reinforcement Learning for Solving the Optimal Policy of the Model in its Abstract Formulation

Based on the abstract formulation in Section 4.1, a policy π in the objective function (15) can be written as πSt=AtdSt,Acθt,St,AtrSt. The purpose is then to find optimal proportionality parameter θ* and for each state S∈S, to identify the optimal control actions A*,dS,A*,cθ*,S,A*,rS, so that the objective function (15) can be minimized. Notice that, if θ is the only parameter for the control actions, then DPG can be used to search for an optimal θ. On the other hand, if we only have to search for the optimal discrete actions A*,dS, then we can employ TD. Since we are simultaneously targeting at these two tasks, we alternatively perform these two optimization steps. We take into account the constraints DS, so that at the TD step, we search over the constraint DdS on the discrete action space Ad, while at the policy gradient step, we project the gradient updates to the continuous constraint DcS on Ac using proximal projection operators (see [72]). We employ on-policy methods rather than off-policy to deal with constraints that are variable with respect to change of states. This enables more exploration over the variable constraints.

Our proposed method is model-free (see [67]), as compared with model-based methods (see [73]). This means that we do not have to know in advance any of the MDP system parameters. Rather, the exploration mechanism in the reinforcement learning algorithm will help us to estimate in an implicit way all the system parameters and make use of them. In our case study in Section 4, the model parameters are given only in order to simulate the system, but the learning algorithm does not require to know these parameters.

Below we write the proposed algorithm into pseudo-code at Algorithm 1.

Algorithm 1 Reinforcement Learning via integrating TD control and DPG with Proximal Projection

1: Input: Input state space S, action space A, constraints Dd,Dc; Given the neural network architecture QS,Ad,Acθ,Ar;ω; Discount factor 0<γ<1; Learning rates ηθ,ηω>0;

2: Initialization: Initialize the weight vector ω0∼ a given prior distribution; initial action A0d,A0r and action parameter θ0, A0c=Acθ0; Initial state S0=S0d,S0c;

3: for t=1,2,…,T do

4: 	Run one step of the MDP from state St under action At=Atd,Atc,Atr, obtain a new state St+1;

5: 	Calculate the total cost ESt,At;

6:	Identify

At+1d=argminAd∈DdSt+1QSt+1,Ad,Acθt,Atr;ωt ;

7:	Based on At+1d, update the policy parameter θ according to DPG (53) and (54):

θt+1=proxDcSt+1,At+1dθt-ηθ∇θAcθt∇AcQSt,Atd,Atc,Atr;ωt ;

8:	Based on At+1d and θt+1, obtain At+1r, so that

At+1=At+1d,At+1c=Acθt+1,At+1r ;

9:	Calculate TD (51):

δt=ESt,At+γQSt+1,At+1;ωt-QSt,At;ωt ;

10:	Update the weight vector ω of the critic based on (52):

ωt+1=ωt-ηωδt∇ωQSt,At;ωt ;

11: end for

12: Output: With the given optimal ω* and θ* found, for each state S given, output the approximate optimal policy A*,d,Acθ*,A*,r , where

A*,d,A*,r=argminAd,AradmissibleQS,Ad,Acθ*,Ar;ω* .

The flow chart of Algorithm 1 is shown in Figure 4 below.

Figure 4: The flow chart of Algorithm 1.

## 4.4 Practical Implementation Details of Algorithm 1 at Original MDP Model

When implementing Algorithm 1 at the original MDP model for the microgrid-manufacturing system, practical issues need to be addressed arise. The set of admissible parameters θ∈DcS,A for continuous actions Ac, that are determined by (S1)-(S6) in Supplementary Material 7.3, have an intersection structure. Indeed from (S1)-(S6) one can view DcS,A as an intersection DcS,A=Dc∩DSOCθ,SOCt,δtb,δtp,δtpb,δtsb,δtbm. Here we set the fixed simplex

| Dc= {λsm,λsb,λwm,λwb,λgm,λgb: λsm,λsb,λwm,λwb,λgm,λgb≥0, 0≤λgm+λgb≤1, 0≤λsm+λsb≤1 ,0≤λwm+λwb≤1} | (55) |
| --- | --- |

The complicacy in DcS,A lies in the other part of the intersection DSOCθ,SOCt,δtb,δtp,δtpb,δtsb,δtbm, that may vary according to the choice of δtb,δtp,δtpb,δtsb,δtbm and depend on the SOC state, which makes the proximal projection to DcS,A in Step 7 of Algorithm 1 nearly impossible to compute in practice. To fix this issue, we suggest to relax the constraint DcS,A by only considering its fixed simplex part Dc. Of course, if we only project to Dc at every proximal projection step in our Step 7 of Algorithm 1, we may miss the SOC constraints in (S1)-(S6). But we can then fix this issue by using the θ found on the relaxed constraint set Dc and determine the binary variables δtb,δtp,δtpb,δtsb,δtbm, as well as update the SOC. If the SOC values we obtain violate the additional constraints in DSOCθ,SOCt,δtb,δtp,δtpb,δtsb,δtbm, we will just set them to be the boundary values SOCmax or SOCmin, so that they will not violate the SOC constraints. In this way, we can find an approximate solution to the optimal one, with computationally achievable simulations.

Under this simplification, the solution algorithm for the original model becomes practical to implement. We can then change Algorithm 1 accordingly, so that

- In Step 6 of Algorithm 1, we select one action At+1d that only takes into account the constraint (30) and minimizes the Q-value. The admissible actions Ad are stored in a tree constructed based on (30) and we search over the tree to identify optimal action At+1d based on Step 6. The action At+1d does not include the indicator variables δt+1b,δt+1p,δt+1pb,δt+1sb,δt+1bm in (43).

- In Step 7 of Algorithm 1, the constraint DcSt+1,At+1d is replaced by a fixed constraint Dc in (55). Notice that in this case, the projection onto Dc can be calculated directly, see Supplementary Material 7.1.

- Once θ is chosen, we determine whether or not we should choose pt+1m+pt+1b≠0, and whether pt+1b≠0 according to (35), (36), and (37); we also determine whether or not we choose bt+1m≠0 according to (34). The precise scheme of choosing pt+1m,pt+1b,bt+1m can be found in Supplementary Material 7.2.

- With all these ready, Step 8 in Algorithm 1 will be modified accordingly.

- At each loop, the optimal actions are implemented for the MDP system to jump to the next state and the incurred cost, throughput and energy demand are calculated.

# 5. Case Study

5.1 Experimental Parameters

5.1.1 Joint Microgrid-Manufacturing System Parameters

In this section, numerical case studies are implemented to illustrate the benefits of the proposed modeling and solution strategies. The open-source code for the proposed model in this paper is released at the GitHub repository [74].

We have carried out experiments of the microgrid-manufacturing system using a real-case parameter set. The manufacturing system includes 5 machines and 4 buffers as shown in Figure 2. We summarize all the system parameters in Tables 1-5. The parameters of the manufacturing system are taken according to [78], where machine and buffer parameters of the manufacturing system are shown in Table 1 and Table 2, respectively. Note that the mean time between failures of each machine is modeled as random variables following Weibull distribution with respective scale and shape parameters. The mean time to repair of each machine is modeled as exponentially distributed random variables. Unit production reward rp is set to be $104 per unit.

Table 1. Machine Parameters

|  | Mean time between failures Scale/Shape Parameter | Mean time to repair (min) | Rated power of operation (kW) | Power at idle state(kW) |
| --- | --- | --- | --- | --- |
| M1 | 111.39 min/1.5766 | 4.95 | 115.5 | 105 |
| M2 | 51.1 min/1.6532 | 11.7 | 115.5 | 105 |
| M3 | 110.9 min/1.7174 | 15.97 | 115.5 | 105 |
| M4 | 239.1 min/1.421 | 27.28 | 170.5 | 155 |
| M5 | 122.1 min/1.591 | 18.37 | 132 | 120 |

Table 2. Buffer Parameters

|  | B1 | B2 | B3 | B4 |
| --- | --- | --- | --- | --- |
| Capacity | 1000 | 1000 | 1000 | 1000 |
| Initial | 100 | 100 | 100 | 100 |

Table 3. Wind Turbine Parameters

| Parameters | Value | Parameters | Value |
| --- | --- | --- | --- |
| vci (m/s)-Cut in speed | 3 | ηt-Gearbox efficiency | 0.9 |
| vco (m/s)-Cut off speed | 11 | ηg-Generator efficiency | 0.9 |
| vr (m/s)-Rate speed | 7 | θ-Power coefficient | 0.593 |
| ρ (kg/m3)-Air density | 1.225 | romcw ($/kWh) | 0.08 |
| r (m)-Blade radius | 25 | Nw (unit) | 1 |

Table 4. Battery Storage Parameters:

| Parameters | Value | Parameters | Value |
| --- | --- | --- | --- |
| e (kWh)-Capacity | 350 | b-Charging rate (kW) | 2 |
| SOCmax (%) | 95 | SOCmin (%) | 5 |
| romcb ($/kWh) | 0.9 | η-Efficiency | 0.99 |

Table 5. Solar Panel and Generator Parameters

| Parameters | Value | Parameters | Value |
| --- | --- | --- | --- |
| a m2-Panel area | 1400 | ng (unit)-Number of generator | 1 |
| δ-Efficiency | 0.2 | Gp (kW)-Generator capacity | 650 |
| romcs ($/kWh) | 0.17 | romcg ($/kWh)-Operating cost | 0.45 |

The parameters of the microgrid used in the experiment are sized based on the manufacturing load according to the methods proposed in [45]. The parameters related to wind turbine, battery storage system, and solar panel and generator are illustrated in Table 4, Table 5, and Table 6, respectively. The data of solar irradiance and wind speed are collected from Solar Energy Local [76] and State Climatologist of Illinois [77], respectively.

In order to prevent computational overflow, in our actual numerical experiment we scaled all the parameters by choosing different units of measurement, with distance measured by km (103 m), time measured by hour (60 min= 3600s), speed measured by km/h (3.6 m/s), energy measured by MegaWatt (106 Watt), money cost measured by 104 US$, area measured by km​2 (106 m​2), mass measured by 106 kg. Time period is measured in hours.

5.1.2 Reinforcement Algorithm Parameters

The neural network QS,A;ω is taken to be fully connected and it contains two hidden layers with 100 neurons for each layer, with Sigmoid and ReLU activations for layers 1 and 2. The output is then scaled back to usual units of measurement, like $ for cost and kW for energy demand.

We take the discount factor γ=0.999. The experiment includes reinforcement learning training for 5×103 iterations, with each iteration counts for time period with equal duration of one hour per period, and the learning rates are tuned to be ηθ=0.003 and ηω=0.0003, with ηω discounted by a factor 0.999 at each iteration. Figure 5-(a) plots the L2-norm of the difference at two consecutive iterations during training in the neural network weights (∥ωt+1-ωt∥22), which clearly indicates the convergence of the training process. Figure 5-(b) plots the cumulative reward function (total incurred cost CS,π in (15) truncated at the current iteration step).

Figure 5. Left to Right: (a) Evolution of weight differences ∥ωt+1-ωt∥22; (b) Evolution of cumulative rewards; (c), (d) Compare with vanilla Q-learning.

In order to validate the effective convergence of our reinforcement learning algorithm, comparison has been carried out for a pure Q-learning algorithm for the same microgrid-manufacturing system with a smaller size (2 machines and 1 buffer) [7]. After discretization of the continuous states and actions, the state space has a size of 3.8×103 and the action space has a size of 2.6×104. We use a smaller fully-connected neural network QS,A;ω with two hidden layers and Sigmoid activation, where each hidden layer has 32 neurons. Again, we calculated the square norms of the differences of the neural-network weight vectors for each two consecutive algorithm iterations (i.e., ∥ωt+1-ωt∥22). The discount factor γ=0.1. The neural network is trained using Adam [75] with different learning rates: Figure 5-(c) is for learning rate 0.001 and Figure 5-(d) is for learning rate 0.0001. It is seen that in these two cases, even after 104 iterations, the vanilla Q-learning algorithm cannot converge due to the immense size of the discretized state-action spaces (a manifestation of the “curse of dimensionality"), indicating the effectiveness of our method that combines deterministic policy gradient with discrete Monte-Carlo type searches. Based on the optimal parameters ω*,θ* found for the neural-network QS,A;ω* and the continuous action Acθ* (see Algorithm 1), we tested the corresponding microgrid-manufacturing model at a time horizon of 100 time periods, with each time period equals one hour.

5.1.3. Baseline Scenarios

The results are compared with two baseline scenarios: The first one runs under a random policy, while the second one is a routine policy.

For the random policy, the accumulation of total incurred cost ES,A at (16), total energy cost TFS,A at (17) and total production units prt in (26) for the optimal policy and random policy are calculated for the system running at a total horizon of 100 time periods (each period =1 hour).

For the routine policy, we consider a routine practice strategy that can be adopted by many industrial practitioners, i.e., the production system and microgrid are controlled or scheduled separately. The production scheduling is generated to minimize total energy consumption without sacrificing target production. This model is briefly introduced as follows. Let xit be the binary decision variable denoting the production schedule of the manufacturing system, i.e., it takes the value of one when machine i is scheduled for production in period t, and zero otherwise. The objective function can be formulated as:

| minxitt∈Txit⋅pi⋅Δt | (56) |
| --- | --- |

where T is the set including all time periods t, pi is the rated power of machine i, Δt is the time duration of each discretization period. Note that for simplicity, the values of PCiOpr are used as the rated power without considering the difference between PCiOpr and PCiIdl.

Two constraints are formulated as follows:

| t∈TxNt⋅PRN≥TA | (57) |
| --- | --- |

where xNt is the decision variable for machine N (i.e., the last machine) of the production system. PRN is the production rate of machine N. TA is the target production count. This constraint shows that the target production should be satisfied. Note that this constraint is based on a simplified assumption that machine breakdown and the resultant blockage/starvation are not considered.

| 0≤Bit+1=Bit+xit⋅PRi-xi+1t⋅PRi+1≤Ci | (58) |
| --- | --- |

where Ci is the capacity of buffer i. Bit+1 is the count of work-in-progress parts stored in buffer location i at the beginning of period t+1. This constraint shows material flow balance and the work-in-progress part in each buffer location cannot exceed its respective bounds.

After solving the aforementioned Integer Programming, the production schedule that can minimize energy consumption without sacrificing production can be obtained. Then, the utilization of microgrids follows the following empirical rules will be implemented. First, the battery storage system and generator are typically considered backups for emergency situations in practice, and thus are not used in this routine policy.

Second, if the renewable sources are available at time period t, they will be first used to satisfy the energy demand of production. If the renewable sources have a higher supply capability than production demand at period t, the remaining part will be sold back to the grid. If the renewable sources have a less supply capability than production demand at period t, the demand gap will be filled by purchasing electricity from the grid. The wind energy has a higher priority to be used than solar source since the cost of wind energy is typically lower than solar energy.

To match the target production unit made by optimal policy, for this routine strategy found by integer programming, we set the target output at the time horizon 100, i.e., the target production unit TA in (56) to be equal to 73, which is the total production throughput in units for the optimal policy (the quantity prt in (26)) at time horizon 100.

5.2 Result Analysis

5.2.1 Random Strategy

The system under randomly chosen policy starts with the same initial conditions as the system under optimal policy. The results of 3 experiments are shown in Figure 6, where red solid lines are for the optimal policy selected by reinforcement learning and black star lines are for the random policy. It is clearly seen from these results that under the optimal policy the manufacturing system tends to produce more throughput with less total cost and similar or less total energy cost (energy demand). More precisely, in one of the experiments, the optimal policy found by reinforcement learning over a time horizon of 100 time periods has an output (the quantity prt in (26) of 73, while the randomly selected policy only produces 24. At the same time, the total cost for the optimal policy is -$728,293 and the total cost for the random policy is -$236,214 . In terms of energy cost, optimal policy is also about one time less than the random policy, with $876 for optimal policy and $1,656 for random policy. The two other experiments behave very similarly.

5.2.2 Routing Strategy

We found that the total energy cost under the optimal policy found by this integer programming is $3,642 . This has been about four times of our previously announced $876 for the optimal policy found by reinforcement learning. Two other experiments are made and the results are similar. The results of the evolution of the total energy cost and total production throughput in units as a function of time are shown in Figure 6, where red solid lines are for the optimal policy selected by reinforcement learning and blue dashed lines are for the routine strategy found by an integer programming. It is clearly seen that the reinforcement learning model selects a policy that incurs less energy cost.

5.2.3 Comparisons

The overall comparison between proposed reinforcement learning model and random policy as well as routine policy is summarized in Table 6, where the last column also compares the average training time (CPU time over 3 experiments) consumed in training for the routine strategy and our proposed reinforcement learning method in the paper.

Table 6. Comparison Among Three Models

|  | Energy Cost ($) | Production Throughput (unit) | Training CPU Time (s) |
| --- | --- | --- | --- |
| Routine Strategy | 3,642 | 73 | 29 |
| Proposed Model | 876 | 73 | 457 |
| Random Policy | 1,656 | 24 | N/A |

Figure 6. Left to Right: Average over 3 experiments the comparison of (a) total throughput in production unit; (b) total energy cost incurred by optimal, routine and random policies: red solid line = optimal policy, blue dashed line = routine strategy via integer programming, black star line = random policy.

# 6. Conclusion

This paper proposes a joint dynamic control model for microgrids and manufacturing systems using Markov Decision Process (MDP) to identify an optimal control strategy for both microgrid components and manufacturing system so that the energy cost for production can be minimized without sacrificing production throughput. A novel reinforcement learning algorithm that leverages both TD and DPG algorithms is proposed to solve the joint control of microgrid and manufacturing system towards cost optimality. Experiments for a manufacturing system with an onsite microgrid with renewable sources have been implemented and the results show the effectiveness of the proposed method in addressing the “curse of dimensionality" in dynamic decision-making with high dimensional and complicated state and action spaces.

There are several remarks and future directions that we would like to address here: (1) One can consider real time decision making for emergency situations such as natural disasters that lead to non-availability of external energy supplies from grids (i.e., some new relevant constraints need to be added); (2) Our method could also be extended to other types of manufacturing systems, e.g., flexible manufacturing system, for an extended application scope; (3) The time consumed for the training process in reinforcement learning can be reduced by using reinforcement learning schemes with more exploration, such as ε-greedy [67], Monte-Carlo tree search [79] or upper-confidence-bound method [80] .

## 7. Supplementary Material

## 7.1 The Projection onto the Simplex Dc

In this section we calculate directly the projection operator proxDcθ for a given θ∈R6 onto the simplex

Dc=Dsc×Dwc×Dgc ,

where Dsc, Dwc, Dgc are three simplices all isomorphic to

D={λ1,λ2:λ1≥0,λ2≥0,0≤λ1+λ2≤1} .

Let θ=θs,θw,θg where each θs,θw,θg∈R2. Then we have

proxDcθ=proxDscθs,proxDwcθw,proxDgcθg .

If θ0∈D, then proxDθ0=θ0. For each θ0=θ01,θ02∈R2∖D, we can easily calculate proxDθ0 according to the following rules

- θ01<0, θ02<0, then proxDθ01,θ02=0,0;

- 0≤θ01≤1, θ02<0, then proxDθ01,θ02=θ01,0;

- θ01>1 and θ01-θ02>1, then proxDθ01,θ02=1,0;

- θ01,θ02>0, θ01+θ02≥1 and -1≤θ01-θ02≤1, then proxDθ01,θ02=1+θ01-θ022,1-θ01+θ022;

- θ02>1 and θ01-θ02<-1, then proxDθ01,θ02=0,1;

- 0≤θ02≤1, θ01<0, then proxDθ01,θ02=0,θ02.

## 7.2 The Scheme for Choosing Battery Charging/Discharging Actions

Once θ is chosen, we determine whether or not we should choose pt+1m+pt+1b≠0, and whether pt+1b≠0 according to (35), (36), and (37); we also determine whether or not we choose bt+1m≠0 according to (34). To be precise, we first calculate the vector

T=(1st+1sb+wt+1sb+gt+1sb>0 , 1st+1b+wt+1b+gt+1b>0,1{SOCt+1-bΔt/η-SOCmin≥0}) ,

where st+1sb, wt+1sb and gt+1sb, st+1b, wt+1b and gt+1b can be calculated according to θt+1 updated in Step 7 of Algorithm 1, and SOCt+1 is from St+1. Then, we determine pt+1m, pt+1b and bt+1m according to Table S1.

Table S1. Determine the values of three parameters

| T | 1,1,1 | 1,1,0 | 1,0,1 | 1,0,0 | 0,1,1 |
| --- | --- | --- | --- | --- | --- |
| pt+1m | 0 | 0 | 0 | 0 | pt+1/0 |
| pt+1b | 0 | 0 | 0 | 0 | 0/pt+1 |
| bt+1m | 0 | 0 | 0/bΔt | 0 | 0 |
| T | 0,1,0 | 0,0,1 | 0,0,1 | 0,0,0 |
| pt+1m | pt+1/0 | pt+1/0 | pt+1 | pt+1/0 |
| pt+1b | 0/pt+1 | 0/pt+1 | 0 | 0/pt+1 |
| bt+1m | 0 | 0 | bΔt | 0 |

Here 0/bΔt means that we randomly pick 0 or bΔt and

pt+1=Et+1mfg-st+1m+wt+1m+gt+1m⋅1{Et+1mfg-st+1m+wt+1m+gt+1m>0} ,

pt+1=Et+1mfg-st+1m+wt+1m+gt+1m+bΔt⋅1{Et+1mfg-st+1m+wt+1m+gt+1m+bΔt>0} .

The 0/pt+1,pt+1/0 means that we randomly sample pt+1b,pt+1m to make their sum pt+1.

## 7.3 Detailed Constraints on θ and SOC

Based on (43), we can further write the constraints (29) and (40)-(42) into different constraints on θ (the parameter in (38)) and SOC. These constraints are listed as follows.

- δtb,δtp,δtpb,δtsb,δtbm=0,0,0,1,1. The constraints on θ are given by

| λsb=λwb=λgb=00≤λsm,λwm,λgm<1SOCmin≤SOCt-bΔt/η≤SOCmax | (S1) |
| --- | --- |

- δtb,δtp,δtpb,δtsb,δtbm=0,1,0,0,1. The constraints on θ are given by

| λsb=λwb=λgb=0λsm=λwm=λgm=1SOCmin≤SOCt-bΔt/η≤SOCmax | (S2) |
| --- | --- |

- δtb,δtp,δtpb,δtsb,δtbm=0,1,1,0,0. The constraints on θ are given by

| - λsb=λwb=λgb=0λsm=λwm=λgm=1 SOCmin≤SOCt+ηEmfg-stm+wtm+gtm+bΔt⋅1Emfg-stm+wtm+gtm+bΔt>0≤SOCmax | (S3) |
| --- | --- |

- δtb,δtp,δtpb,δtsb,δtbm=1,1,0,0,0. The constraints on θ are given by

| - λsm≥0,λsb>0, λsm+λsb=1λwm≥0,λwb>0,λwm+λwb=1λgm≥0,λgb>0,λgm+λgb=1SOCmin≤SOCt+ηstb+wtb+gtb≤SOCmax | (S4) |
| --- | --- |

- δtb,δtp,δtpb,δtsb,δtbm=1,0,0,1,0. The constraints on θ are given by

| λsm≥0,λsb>0,0≤λsm+λsb<1λwm≥0,λwb>0,0≤λwm+λwb<1λgm≥0,λgb>0,0≤λgm+λgb<1SOCmin≤SOCt+ηstb+wtb+gtb≤SOCmax | (S5) |
| --- | --- |

- δtb,δtp,δtpb,δtsb,δtbm=1,1,1,0,0. The constraints on θ are given by

| λsm≥0,λsb>0,λsm+λsb=1 λwm≥0,λwb>0,λwm+λwb=1λgm≥0,λgb>0,λgm+λgb=1SOCmin≤SOCt+η[stb+wtb+gtb +Emfg-stm+wtm+gtm+bΔt ⋅1Emfg-stm+wtm+gtm+bΔt>0]≤SOCmax | (S6) |
| --- | --- |

## 7.4 Summary of Notations and Symbols

For convenience of the reader, we have summarized all the notations and symbols used in this paper as follows.

Actions and Policy
A: Action space 
π: Policy mapping S→A 
A: An action A∈A 
At: Action at time t 
Atmfg: Action for the manufacturing system 
Atmic: Action for the microgrid system 
ati: Control action for machine i∈{1,...,N} at time t 
K-action: Keep original machine state &
W-action: Turn a machine, which was Off, on 
H-action: Turn a machine Off 
ats,atw,atg: Action of adjusting working status of the solar, wind, and the generators in the microgrid 
stm,stb,stsb: Solar energy used to support manufacturing, charging the battery, and selling back to the grid, respectively
wtm,wtb,wtsb: Wind energy used to support manufacturing, charging the battery, and selling back to the grid, respectively
gtm,gtb,gtsb: Generator energy used to support manufacturing, charging the battery, and selling back to the grid, respectively
ptm,ptb: Energy purchased from the grid for manufacturing and charging the battery, respectively 
btm: Energy discharged by the battery to support manufacturing 
δtbm: Binary variable indicating whether the battery is discharged to support manufacturing or not
b: Positive battery discharge rate

Parameterization of Action Space & Constraints
λsm,λsb,λwm, λwb,λgm,λgb: Proportionality parameters in 0,1, sum in each index less than or equal to 1 
Std: Discrete states at time t 
Stc: Continuous states at time t 
Atd: Discrete actions at time t 
Atc: Continuous actions at time t 
Atr: Remaining actions at time t 
DdStd: Set of admissible discrete actions 
DcStd,Stc,Atd: Set of admissible continuous actions 
δt: On-policy temporal difference 
θ: Parameters for the continuous part of the policy 
xit: Decision variable for machine i at time t 
Ci: Capacity of the i-th buffer

States
S: State space 
St: System state at time t, St∈S 
Stmfg: State of the manufacturing system 
Stmic: State of the microgrid system 
Stenv: State of the environment 
SitM: State of machine i∈{1,...,N} 
SitB: State of buffer i∈{1,...,N-1} 
gts: Working status of solar PV at time t 
gtw: Working status of wind turbine at time t 
gtg: Working status of generator at time t 
SOCt: State of charge of the battery system at time t 
It: Solar irradiance at time t 
vt: Wind speed at time t

General
MDP: Markov Decision Process

TD: Temporal Difference

DPG: Deterministic Policy Gradient

References

[1] C. Mahieux and A. Oudalov. Microgrids Enter the Mainstream, RenewableEnergyFocus.com, http://www.renewableenergyfocus.com/view/43345/microgrids-enterthe-mainstream/, 2015.

[2] U.S. Department of Energy. Technical Report: How Microgrids Work. http://www.energy.gov/articles/how-microgrids-work, 2014.

[3] Lawrence Berkeley National Laboratory, Technical Report: Microgrid Definitions. https://building-microgrid.lbl.gov/microgrid-definitions, 2016.

[4] B. Lasseter. Microgrids [distributed power generation]. In Power Engineering Society Winter Meeting, IEEE, 1: 146-149, 2001.

[5] B. Lasseter. Microgrid. In Power Engineering Society Winter Meeting, IEEE, 1: 305-308, 2002.

[6] W. Hu, Z. Sun, Y. Zhang, and Y. Li. Joint manufacturing and onsite microgrid system control using markov decision process and neural network integrated reinforcement learning. Procedia Manufacturing, 39:1242-1249, 2019.

[7] P. Zeng, H. Li, H. He, and S. Li. Dynamic energy management of a microgrid using approximate dynamical programming and deep recurrent neural network learning. IEEE Transactions on Smart Grid, 10(4): 4435-4445, 2019.

[8] Y. Ji, J. Wang, J. Xu, X. Fang, and H. Zhang. Real-time energy management of a microgrid using deep reinforcement learning. Energies, 12: 2291, 2019.

[9] M.A.A. Faruque. RAMP: Impact of rule based aggregator business model for residential microgrid of prosumers including distributed energy resources. In Proceeding of IEEE PES Innovative Smart Grid Technologies Conference (ISGT), 1-6, 2014.

[10] P.O., Kriett, and M. Salani. Optimal control of a residential microgrid. Energy; 42(1): 321-330, 2012.

[11] L. Roggia, C. Rech, L. Schuch, J.E. Baggio, H.L. Hey, and J.R. Pinheiro. Design of a sustainable residential microgrid system including PHEV and energy storage device. In Proceedings of the 2011 14th European Conference on Power Electronics and Applications, 1–9, 2011.

[12] F. Ahourai and M.A. Al Faruque. Technical Report: Grid Impact Analysis of a Residential Microgrid under Various EV Penetration Rates in GridLab-D. Center for Embedded Computer Systems, Irvine, CA, 2013.

[13] L., Igualada, C., Corchero, M., Cruz-Zambrano, and F.J. Heredia. Optimal energy management for a residential microgrid including a vehicle-to-grid system. IEEE Transactions on Smart Grid; 5(4): 2163-2172, 2014.

[14] A.D. Hawkes and M.A. Leach. Cost-effective operating strategy for residential micro-combined heat and power. Energy, 32(5): 711-723, 2007.

[15] M. Tasdighi, H. Ghasemi, and A. Rahimi-Kian. Residential microgrid scheduling based on smart meters data and temperature dependent thermal load modeling. IEEE Transactions on Smart Grid; 5(1): 349-357, 2014.

[16] H. Kakigano, Y. Miura, T. Ise, T. Momose, and H. Hayakawa. Fundamental characteristics of DC microgrid for residential houses with cogeneration system in each house. In Proceedings of IEEE Power Energy Society General Meeting-Convers. Del. Elect. Energy 21st Century, 18, 2008

[17] D.E. Olivares, A. Mehrizi-Sani, A.H. Etemadi, C.A. Canizares, R. Ira-vani, M. Kazerani, A.H. Hajimiragha, O. Gomis-Bellmunt, M. Saeedifard, R. Palma-Behnke, G.A. Jimenez-Estevez, and N.D. Hatziargyriou.  Trends in microgrid control. IEEE Transactions on Smart Grid, 5(4):1905–1919, 2014.

[18] P. Malysz, S. Sirouspour, and A. Emadi, An optimal energy storage control strategy for grid-connected microgrids, IEEE Trans. Smart Grid, 5(4): 1785-1796, 2014.

[19] A. Parisio, E. Rikos, and L. Glielmo. A model predictive control approach to microgrid operation optimization, IEEE Transactions on Control Systems Technology, 22(5): 1813-1827, 2014.

[20] M. Petrollese, L. Valverde, D. Cocco, G. Cau, and J. Guerra. Real-time integration of optimal generation scheduling with MPC for the energy management of a renewable hydrogen-based microgrid, Applied Energy, 166: 96-106, 2016.

[21] E. Craparo, M. Karatas, and D. I. Singham. A robust optimization approach to hybrid microgrid operation using ensemble weather forecasts, Applied Energy, 201: 135-147, 2017.

[22] W. Su, J. Wang, and J. Roh. Stochastic energy scheduling in microgrids with intermittent renewable energy resources, IEEE Transactions on Smart Grid, 5(4): 1876-1883, 2014.

[23] F. Farzan, M. A. Jafari, R. Masiello, and Y. Lu. Toward optimal day-ahead scheduling and operation control of microgrids under uncertainty, IEEE Transactions on Smart Grid, 6(2): 499-507, 2015.

[24] Z. Li, C. Zang, P. Zeng, and H. Yu. Combined Two-Stage Stochastic Programming and Receding Horizon Control Strategy for Microgrid Energy Management Considering Uncertainty. Energies, 9: 499, 2016.

[25] F.A. Mohamed, and H.N. Koivo. Online management genetic algorithms of microgrid for residential application. Energy Conversion and Management; 64: 562-568, 2012.

[26] M. Ross, R. Hidalgo, C. Abbey, and G. Joós. 2011. Energy storage system scheduling for an isolated microgrid. IET Renewable Power Generation; 5(2): 117-123, 2011.

[27] J. Proano. Microgrid power flow study in grid-connected and islanding modes under different converter control strategies. In Proceedings of 2012 IEEE Power and Energy Society General Meeting, pp. 18, 2012.

[28] H. Yang, F. Wen, and L. Wang. Newton-Raphson on power flow algorithm and Broyden method in the distribution system. In Proceedings of PECon, IEEE 2nd International Power and Energy Conference, pp. 1613-1618, 2008.

[29] J. Driesen, J., and K. Visscher. Virtual synchronous generators. In Proceedings of IEEE Power Energy Society General Meeting Convers. Del. Elect. Energy 21st Century, pp.13, 2008.

[30] G. Diaz, C. Gonzalez-Moran, J. Gomez-Aleixandre, and A. Diez. Scheduling of droop coefficients for frequency and voltage regulation in isolated microgrids. IEEE Transactions on Power System, 25(1): 489-496, 2010.

[31] A. Sadegheih. Optimal design methodologies under the carbon emission trading program using MIP, GA, SA, TS. Renewable and Sustainable Energy Reviews; 15(1): 504-513, 2011.

[32] A. Chaouachi, R.M. Kamel, R. Andoulsi, and K. Nagasaka. Multiobjective intelligent energy management for a microgrid. IEEE Transactions on Industrial Electronics; 60: 1688-1699, 2013.

[33] J.R. Duflou, J.W. Sutherland, D. Dornfeld, C. Herrmann, J. Jeswiet, S. Kara, M. Hauschild, and K. Kellens. Towards energy and resource efficient manufacturing: A processes and systems approach. CIRP Annals-Manufacturing Technology, 61(2): 587-609, 2012.

[34] U.S. Department of Energy. Annual Energy Review 2009. ftp://ftp.eia.doe.gov/multifuel/038409.pdf, 2010.

[35] S. Ma, Y. Zhang, Y. Liu, H. Yang, J. Lv, and S. Ren. Data-driven sustainable intelligent manufacturing based on demand response for energy-intensive industries, Journal of Cleaner Production, 274: 123155, 2020.

[36] Y. Zhang, S. Ma, H. Yang, J. Lv, and Y. Liu. A big data driven analytical framework for energy-intensive manufacturing industries, Journal of Cleaner Production, 197: 52-72, 2018.

[37] J. Lv, R. Tang, and S. Jia. Therblig-based energy supply modeling of computer numerical control machine tools, Journal of Cleaner Production, 65: 168-177, 2014.

[38] L. Yun, S. Ma, L. Li, and Y. Liu. CPS-enabled and knowledge-aided demand response strategy for sustainable manufacturing. Advanced Engineering Informatics 52, 101534, 2022.

[39] L. Yun, L. Li, and S. Ma. Demand response for manufacturing systems considering the implications of fast-charging battery powered material handling equipment. Applied Energy 310, 118550, 2022.

[40] Z. Sun, L. Li, M. Fernandez, and J. Wang. Inventory control for peak electricity demand reduction of manufacturing systems considering the tradeoff between production loss and energy savings. Journal of Cleaner Production, 82: 84-93, 2014.

[41] Z. Sun, F. Dababneh, and L. Li. Joint energy, maintenance, and throughput modeling for sustainable manufacturing systems. IEEE Transactions on Systems, Man, and Cybernetics: Systems, 50(6): 2101-2112, 2020.

[42] F. Katiraei, C. Abbey, S. Tang, and M. Gauthier. Planned islanding on rural feeders—utility perspective. In Proceedings of Power and Energy Society General Meeting-Conversion and Delivery of Electrical Energy in the 21st Century, IEEE, pp.1-6, 2008.

[43] J. Turkewitz. Unemployment Deepens Storm’s Loss as Businesses Stay Closed. http://www.nytimes.com/2012/12/28/nyregion/unemploymentdeepens-the-loss-from-hurricane-sandy.html? r=0, 2012.

[44] M.M. Islam, Z. Sun, and X. Yao. Simulation-based investigation for the application of microgrid with renewable sources in manufacturing systems towards sustainability. In ASEM 2016 International Annual Conference, Charlotte, NC, USA. American Society for Engineering Management, 2016.

[45] M.M. Islam and Z. Sun. Onsite generation system sizing for manufacturing plant considering renewable sources towards sustainability. Sustainable Energy Technologies and Assessments, 32:1–18, 2019.

[46] X. Zhong, M.M. Islam, H. Xiong, and Z. Sun. Design the capacity of onsite generation system with renewable sources for manufacturing plant. In Procedia Computer Science, Complex Adaptive Systems Conference, Chicago, IL, USA, volume 114, pages 433–440, 2017.

[47] V. Santana-Viera, J. Jimenez, T. Jin, and J. Espiritu. Implementing factory demand response via onsite renewable energy: A design-of-experiment approach. International Journal of Production Research, 53(23): 7034-7048, 2015.

[48] T.J. Harper. A Novel Microgrid Demand-Side Management System for Manufacturing Facilities. Master thesis, 2014. Purdue University. http://docs.lib.purdue.edu/techmasters/85/

[49] T.J. Harper, W.J. Hutzel, A. Kulatunga, J.C. Foreman, and A.L. Adams. Microgrids for Improving Manufacturing Energy Efficiency. In Proceedings of International High Performance Buildings Conference, 2014.

[50] M. Golari, N. Fan, and T. Jin, Multistage Stochastic Optimization for Production-Inventory Planning with Intermittent Renewable Energy, Production and Operations Management, 26(3): 409-425, 2017.

#### [51] Reducing energy use by the pulp and paper industry will require greater recycling and waste heat recovery, IEA, https://www.iea.org/fuels-and-technologies/pulp-paper

[52] A.K. Pandey, and R. Prakash, Energy Conservation Opportunities in Pulp & Paper Industry, Open Journal of Energy Efficiency, 7(4), 2018.

[53] Ashok, S. Peak-Load Management in Steel Plants. Applied Energy, 83(5): 413-424, 2006.

[54] Ashok, S. and R. Banerjee. An Optimization Mode for Industrial Load Management. IEEE Transactions on Power Systems, 16(4): 879-884, 2001.

[55] S. Ma, Y. Zhang, J. Lv, H. Yang, and J. Wu. Energy-cyber-physical system enabled management for energy-intensive manufacturing industries. Journal of Cleaner Production, 226: 892-903, 2019.

[56] S. Ma, Y. Zhang, J. Lv, Y. Ge, H. Yang, and L. Li. Big data driven predictive production planning for energy-intensive manufacturing industries. Energy, 211:118320, 2020.

[57] S. Ma, Y. Zhang, S. Ren, H. Yang, and Z. Zhu. A case-practice-theory-based method of implementing energy management in a manufacturing factory. International Journal of Computer Integrated Manufacturing, 34: 829-843. 2021.

[58] H. Zhang, W. Sun, W. Li, and Y. Wang. Physical and chemical characterization of fugitive particulate matter emissions of the iron and steel industry, Atmospheric Pollution Research, 13: 101272, 2022.

[59] W. Sun, Q. Wang, Y, Zhou, and J. Wu. Material and energy flows of the iron and steel industry: Status quo, challenges and perspectives, Applied Energy, 268: 114946, 2020.

# [60] H. Zhang, W. Sun, W. Li, and G. Ma. A carbon flow tracing and carbon accounting method for exploring CO2 emissions of the iron and steel industry: An integrated material–energy–carbon hub. Applied Energy, 309: 118485, 2020.

[61] C. Meirina, Y. N. Levchuk, G. M. Levchuk, and K. R. Pattipati. A markov decision problem approach to goal attainment. IEEE Transactions on Systems, Man, and Cybernetics - Part A: Systems and Humans, 38(1):116-132, 2008.

[62] P. Doshi, X. Qu, A. S. Goodie, and D. L. Young. Modeling human recursive reasoning using empirically informed interactive partially observable markov decision processes. IEEE Transactions on Systems, Man, and Cybernetics - Part A: Systems and Humans, 42(6):1529-1542, 2012.

[63] E. Kuznetsova, Y. F. Li, C. Ruiz, E. Zio, G. Ault, and K. Bell. Reinforcement learning for microgrid energy management. Proc. of the 35th International Conference on Uncertainty in Artificial Intelligence (UAI), Tel Aviv, Israel, 59:133-146, 2013.

[64] Y. Dallery. On modeling failure and repair times in stochastic models of manufacturing systems using generalized exponential distributions. Queueing Systems, 15(1-4):199-209, 1994.

[65] Y. Wang and L. Li. A novel modeling method for both steadystate and transient analyses of serial bernoulli production systems. IEEE Transactions on Systems, Man, and Cybernetics: Systems, 45(1):97-108, 2014.

[66] J. Li and S.M Meerkov. Production systems engineering. Springer Science & Business Media,

[67] R.S. Sutton and A.G. Barto. Reinforcement Learning: An Introduction. Second Edition, in progress, complete draft online. MIT Press, November 5, 2017.

[68] V.R. Konda and J.N. Tsitsiklis. Actor-Critic Algorithms. Neural Information and Processing Systems, 2000.

[69] V. Mnih, K. Kavukcuoglu, D. Silver, A. Graves, I. Antonoglou, D. Wierstra, and M. Riedmiller. Playing atari with deep reinforcement learning. Neural Information and Processing Systems (NIPS), 2013.

[70] D.P. Bertsekas and J.N. Tsitsiklis. Neuro–Dynamic Programming. Athena Scientific, 1996.

[71] D. Silver, G. Lever, N. Heess, T. Degris, D. Wierstra, and M. Riedmiller. Deterministic policy gradient algorithms. ICML (International Conference on Machine Learning), 2014.

[72] N. Parikh and S. Boyd. Proximal algorithms. Foundations and Trends in Optimization, 1(3):123-231, 2013.

[73] M. Janner, J. Fu, M. Zhang and S. Levine, When to Trust Your Model: Model-Based Policy Optimization, Neural Information and Processing Systems, 2019.

[74] Github-Hu, 2020, Source code for our paper: https://github.com/huwenqing0606/rlmanufacturing.

[75] L. Li and Z. Sun. Dynamic energy control for energy efficiency improvement of sustainable manufacturing systems using Markov Decision Process. IEEE Transactions on Systems, Man, and Cybernetics: Systems, 43: 1195-1205, 2013.

[76] Solar Energy Local: Solar energy data and resources in the us. https://solarenergylocal.com/.

[77] State Climatologist Office for Illinois. http://www.isws.illinois.edu/atmos/statecli/wind/wind.htm.

[78] D.P. Kingma and J. Ba. Adam: A method for Stochastic Optimization. ICLR, arXiv:1412.6980[cs.LG], 2015.

[79] C. Browne, E. Powley, D. Whitehouse, S. Lucas, P.I. Cowling, P. Rohlfshagen, S. Tavener, D. Perez, S. Samothrakis, and S. Colton. A Survey of Monte Carlo Tree Search Methods. IEEE Transactions on Computational Intelligence and AI in Games, 4(1): 1-43, 2012.

[80] P. Auer. Using Confidence Bounds for Exploitation-Exploration Tradeoffs. Journal of Machine Learning Research, 3: 397-422, 2002.